Use & Care of 3D Floodplain Model


Floodplain Model to be used ONLY BY CERTIFIED OFMA members who have successfully completed the training. 
Presenter is responsible for care and condition of the model.

Unpacking the Model:

· Carefully remove all components from the crate.  Save all packing materials.

· Refer to Users Guide (page 4) for assembly and setup of the model.

Repacking the Model:

· Using only a soft clean towel dry all components. 

· Squeeze out all sponges.

· Store damp towels and sponges in one of the 5 gallon buckets outside the crate.

· Remove drain hose and fitting from watershed tank.

· Carefully place dry acrylic watershed tank in the crate.  CAUTION:  the acrylic watershed tank is easily scratched.

· Wrap the other dry acrylic components in the bubble wrap provided and place inside watershed tank.

· Place other small components in other 5 gallon bucket and put in the acrylic watershed tank.

Storing the Model after use.  *IF MODEL STORED DAMP IT WILL MILDEW – SEVERE DAMAGE WILL RESULT.

· Leave damp towels and sponges in bucket outside the crate for several days.

· Prop lid of crate open slightly to allow residual dampness to evaporate.
ONLY USE a soft cloth and water for cleaning.

NEVER use cleaning solutions or solvents on the acrylic as this may cause cracking and crazing to occur.
NEVER use any abrasive cleaners on the landform model – this will scratch and damage the painted fiberglass surface.

NEVER leave water in the model for a prolonged period of time – this will cause staining and mold growth.
